WCS Logger

The assistant in WCS PC host applications

About this document

Scope and purpose

This automation tool assists users in collecting all necessary information / data through Infineon CY4500 CC sniffer and 3rd party log devices, includes Saleae and Ellisys C-Tracker.

Intended audience

Users who adapt Infineon WCS USB Type-C solutions in PC HOST applications and frequently communicate with Infineon WCS support teams in issue debugging discussions.

Table of Contents

About this document	1

1	Introduction	3

1.1	Prerequisites	3

1.2	Installation	4

2	Infineon Automation Tool Package	6

2.1	HPI Analyser	6

2.2	CC Analyser	6

2.3	Infineon WCS Logger	7

3	How to use	10

3.1	Scenario A	10

3.2	Scenario B	11

4	Limitations of Infineon WCS Logger	12

Self Service	13

Revision history	15

# Introduction

Infineon WCS CCG devices are commonly used in PD port controller applications in systems like Notebook and Desktop computers, tablets, mobiles etc. In such cases, the system design typically includes a Host Processor or Embedded Controller (EC) that is responsible for the system policy decisions. The CCG controller can be connected to the EC and the CCG firmware allows the EC to monitor the status of the USB-PD ports, change configurations, perform firmware updates, and transparently interact with other USB-PD devices connected to CCG. All the capabilities are exercised through commands/responses transferred across the Host Processor Interface (HPI).

When seeing issues in the field, it is required to provide relevant information to Infineon WCS support teams to accommodate preliminary diagnosis and appropriate resolutions. Therefore, a comprehensive log package with accurate data is mandatory to accelerate solutions delivery.

Figure 1 shows the standard procedure of debugging flows:

- Standard issue report flow

## Prerequisites

There are plenty of log devices available in the market. Except Infineon self-own CY4500 CC sniffer, there are two 3rd party tools recommended to enhance debugging efficiency:

- Saleae Logic Pro 16 + Saleae Logic2 application v2.4.0 or above

- Ellisys C-Tracker + Ellisys Type-C Tracker Analyzer

Infineon WCS support team creates a tool package to integrate all supported log devices into a single application, alone with advanced plugins to facilitate log collection activities. It includes:

- HPI Analyser plugin for Saleae: a specialised I2C analyser dedicated to Infineon HPI protocol.

- CC Analyser plugin Saleae: USB Type-C vendor define message analyser.

- Infineon WCS Logger: a GUI tool to generate logs with all necessary information.

## Installation

First, go to Google Drive and download the entire “wcs_logger” folder (download here):

- Copy “wcs_logger” folder to arbitrary directory except root directory, which might need additional access permission for file creation later. Current user’s desktop is recommended.

- WCS logger tool package

- Install EZ-PD Protocol Analyzer Utility v3.1.0 (download here).

- Install Saleae Logic2 application (www.saleae.com) and:

  - Enable “Automation” via Preference -> Automation -> Enable Automation Server.

- Saleae Logic2 Preferences – Automation

  - Copy “HPI_regs.xml”, “i2c_analyzer_for_HPI.dll” and “saleae_pd_analyser_release.dll” to the folder assigned in “Custom Low Level Analysers”.

- Saleae Logic2 Preferences – Low Level Analyzers

- Install Ellisys Type-C Tracker Analyzer application (www.ellisys.com) and:

  - Copy “RemoteControl” folder to its installation directory.

  - Confirm Remote Control is setup via Menu -> Tools -> Options -> Remote Control.

- Ellisys Type-C Tracker Analyzer

- Done.

# Infineon Automation Tool Package

## HPI Analyser

The native I2C Analysers in Saleae Logic2 application can decode and list bus messages to its terminal-like window and tabular-format table for search usage. It decodes I2C commands in BYTE format basis and is useful in generic use cases.

However, Infineon HPI protocol is a dual-byte basis as its register address and usually conducts a WORD or longer data length as a single command and responses, and it is not intuitive to translate decoded messages through native I2C analyser. Worse, it is hard to do HPI command search. Given that it causes additional efforts and inconveniences to users, especially for engineers who are dealing with HPI activities in daily basis.

Hence, Infineon WCS support team creates a customised Saleae I2C analyser plugin. It can re-synthesise standard I2C byte packets to scalable-length message format, allows users to change significant byte order and capable to translate HPI raw data. In addition, it is easier to do HPI command analysis by introducing timestamp ahead of every HPI messages.

- Additional options in Infineon “I2C_HPI” analyser

## CC Analyser

USB CC and HPI logs are mandatory information WCS support team usually need while dealing with issues. In the past, they were provided by CY4500 CC sniffer and Saleae Logic Analyser separately, and always was difficult to understand command sequences among CC and HPI activities because these logs are not referring the same timeline.

By introducing Infineon CC Analyser with a proper configuration, users can capture and decode both USB CC and HPI logs through Saleae Logic2 application alone. It is extremely useful to check relevant command sequences across different protocols and accelerate debugging progress.

- Infineon “Saleae_PDCC_Release” Analyser

## Infineon WCS Logger

This is a portable application that supports Microsoft Windows 10 and above. It is highly recommended to use it with Saleae Logic Analyser Pro 16 and Ellisys C-Tracker. The application pre-defines all necessary trace pins that users are required to provide while reporting issues for both AMD and INTEL design cases, along with project-relevant information, such as PD firmware revision. All numbers in pulldown menus are selectable but cannot be duplicated.

- Pin needs per WCS PD Host E2 training

SALEAE/Ellisys log files will be generated based on channel number settings, associated with corresponding bus analysers respectively. These files are named beginning with DATE_TIME as a prefix and appended with selected data in the Version Information section. In addition, an issue tracker form and a text file named by the current date will be outputted under the same naming folder.

The Main window splits in following sections (refer to Figure 9):

6 5 4 2 1

32

Infineon WCS Logger

- Issue Information: it is mandatory to fill all information correspondingly, whereas OEM Ticket# as optional. It also covers both AMD and INTEL design trace needs. All information will be recorded into issue tracker form automatically.

- Generic: this section describes common trace pins need in WCS PC host application. This section will be switched to Ellisys mode while work in “Ellisys C-Tracker” mode (Figure 10):

- Ellisys mode

  - USB-C: it defines SALEAE channel numbers to basic USB Type-C pins. All channels are recorded in digital mode as default, and can be set to digital + analogue mode by enabling the “Analogue Mode” option if necessary.

  - PD/PD to EC: it defines HPI bus to EC and PD UART.

- AMD/INTEL Platform: it defines all necessary trace pins need for AMD and INTEL designs respectively. Please be notices that due to insufficient pin counts available in SALEAE, in AMD designs, MUX power pin SALEAE channel is shared with HPD pin. Please connect either one depends on issue category for AMD design cases.

- Saleae Logic/Ellisys C-Tracker: it supports both log devices and will refresh its selections in real-time. In addition, EZ-PD Protocol Analyzer Utility will incorporate with Saleae Logic2 application when work in “Saleae Logic” mode.

- Start/Abort Recording: the main button to control log applications. It will be grey-out if data missing in Version Information section or channel selection duplication. Use this button to start / stop log capturing processes.

- Pass/Fail Case: the main buttons to save recorded logs for pass and fail cases. By clicking either buttons, it will save the current logs as pass or fail cases and export to an issue tracker form with all data described in Version Information section. It will be grey-out before log capturing.

# How to use

Figure 11 is functional flow chart to illustrate Infineon WCS logger working model. WCS Logger will run into either Saleae or Ellisys mode based on connecting status of both SALEAE Logic Analyzer and Ellisys C-Tracker Analyzer while launching. Please notice that due to log file requirement from Infineon WCS support teams, it is mandatory to connect CY4500 if work in Saleae mode.

Depends on log applications running condition, there are 2 possible scenarios in Infineon WCS Logger usage:

- Infineon WCS Logger functional flow

## Scenario A

This user scenario presumes either Saleae Logic2 or Ellisys C-Tracker application is not running in the background while starting Infineon WCS Logger application. The benefit is users do not need to enable Saleae Automation option beforehand.

- Start Infineon WCS Logger.

- Fill all information and platform type in Issue Information and mapping pins with pre-defined channels or preferred ones.

- Click “Start Recording” to start log capturing per setup.

- Infineon WCS Logger launches log applications with automation supportive.

- Do issue replication steps. (e.g., shutdown and reboot to S0, re-plugging devices again, etc.)

- Click “Abort Recording”, “Pass Case” or “Fail Case” while replication steps finished.

- Infineon WCS Logger will

  - In Saleae mode: assigns bus analysers to the SALEAE log and save. Record pin capture settings in the text configuration file. In additional, a CY4500 log is created and save.

  - In Ellisys mode: save log file and close it.

  - Update tracker form sheet with all recorded log filenames.

- Repeat step 3 to 7 for another log capture round.

- Close Infineon WCS Logger while finished, and log applications will be shut down.

## Scenario B

This user scenario only works while automation option is enabled in Saleae Logic2 application as pre-requisites in Saleae mode. In this case, it is presumed users launch log applications already.

- Start Infineon WCS Logger.

- Fill all information and platform type in Issue Information and mapping pins with pre-defined channels or preferred ones.

- Click “Start Recording” to start log capturing per setup.

- Do issue replication steps. (e.g., shutdown and reboot to S0, re-plugging devices again, etc.)

- Click “Abort Recording”, “Pass Case” or “Fail Case” while replication steps finished.

- Infineon WCS Logger will

  - In Saleae mode: assigns bus analysers to the SALEAE log and save. Record pin capture settings in the text configuration file. In additional, a CY4500 log is created and save.

  - In Ellisys mode: save log file and close it.

  - Update tracker form sheet with all recorded log filenames.

- Repeat step 3 to 6 for another log capture round.

- Close Infineon WCS Logger while finished, and log applications will ***NOT*** be shut down.

# Limitations of Infineon WCS Logger

There are several limitations of Infineon WCS Logger, including:

- It supports Microsoft Windows 10 and above only.

- It might crash if Saleae Logic2 application does not enable automation option while working at Scenario B.

- Controlling log applications directly, includes minimise application windows might cause Infineon WCS Logger crash.

- It does not support Saleae Logic Analyser 8 / Pro 8 due to insufficient channel numbers.

Self Service

| CATEGORY | SYMPTOM | TROUBLE SHOOTING |
| --- | --- | --- |
| Not Start. | WCS Logger does not start and shows one of following dialogs instead. | Please check if either Saleae Logic 16 Pro or Ellisys C-Tracker is being appropriate attached and enumerated on Host systems or not. |
|  |  | Please check if CY4500 is setup as PD3.0 supportive and being attached and enumerated appropriately on Host systems or not. |
|  |  | Please go to Infineon website and download latest EZ-PD Protocol Analyzer Utility installer. It is highly recommended to install this application by its default setups. |
|  |  | Please go to Saleae website and download latest Logic2 installer. It is highly recommended to install this application by its default setups. |
|  |  | Please go to Ellisys website and download latest Ellisys Type-C Tracker Analyzer installer. It is highly recommended to install this application by its default setups. |
|  |  | Please copy the entire “RemoteControl” folder inside WCS Logger package to Ellisys Type-C Tracker Analyzer installation directory. By default, it is located at C:\Program Files (x86)\Ellisys\Ellisys Type-C Tracker Analyzer\. |
| WCS Logger crashes. | WCS Logger crashes after hitting “Start Capturing” | Please check if Saleae Logic2 application is running and did not enable its automation option while launching WCS Logger. |
|  |  | Please check if EZ-PD Protocol Analyzer Utility is stuck at file saving or save confirmation dialog. It is recommended to close all popup dialogs or shutdown EZ-PD Protocol Analyzer Utility before launching WCS Logger. |
|  |  | Please check if latest version of EZ-PD Protocol Analyzer Utility is installed. It is recommended to install v3.1.x with default setups. |
|  |  | Please check if “tracker_form.xlsx” is placed in the same folder with WCS Logger application or not. |
|  |  | Please check if “AnalyzerRemoteControl.ice” is placed in the same folder with WCS Logger application or not. |
|  | WCS Logger crashes after hitting either “Pass case” or “Fail Case” | Please check if WCS Logger is copied to a folder which current user has sufficient permissions to access it. |

Revision history

| Document version | Date of release | Description of changes |
| --- | --- | --- |
| 1.00 | 2023-07-20 | First release. |